# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Procurement of specialized airborne equipment for project AInspec

Ref. number: HR-RS00236 – PP2 – TD04

Columns 1-2 should be completed by the Project partner

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Project partner shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

The requirements set out in the technical specifications represent the minimum technical characteristics which offered goods must satisfy, unless stated otherwise, and tenderers are not allowed to modify technical specification in any way.

For each item for which it is not explicitly stated that it is allowed to offer goods of the equal characteristics, i.e. for each item where it is not stated “or equivalent”, for the purposes of this tender documentation it is assumed that words “or equivalent” are stated, and tenderer is allowed to offer equivalent goods / goods of equivalent characteristics.

General Requirements

•	All requirements stated and outlined in this document must be regarded as mandatory and the minimum acceptable criteria. All requirements outlined in this document are accompanied by the phrase "or equivalent".

•	The tenderer is required to provide the specifications of the offered items in the Technical Offer, including details such as the manufacturer, product type, model, and country of origin, and all the other relevant documents as per specification required. All documentation must be provided in English or Serbian, both in hardcopy and electronic formats.

•	The selected contractor is required to provide all necessary documentation for the registration of the completed vehicle in accordance with the relevant legal regulations in the Republic of Serbia.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s decision (Y/N) |
| --- | --- | --- | --- | --- |
| 1 | Drone system for a hyperspectral/multispectral imaging Quantity: 1 Unit set (Unmanned treatment system specialised for forestry) |  |  |  |
|  | Required contents and characteristics of the Unit set: • Enterprise grade Drone Aircraft system, capable of running accompanied hyperspectral sensor in mapping missions, with Remote Controller, hardcase and standard accessories, incl. auxiliary RC battery and RC strap • Full flight battery set for drone (3 intelligent flight batteries) • Manufacturers approved Battery Station for charging • Hyperspectral/multispectral imaging camera w. 90 spectral channels in 400-1000nm wavelength range, compatible w. Drone Aircraft system, with license for hyperspectral/ multispectral Postprocessing software • Certified installation of the system and technical support by official manufactures distributor and maintenance provider with at least 2 certified solution engineers • Certified training in the application of the system for imaging in forestry, provided by 2 trainers w. official instructors certification of the Drone manufacturers Academy, w. official aviation instructors competency by EASA / ICAO in domain of professional UAS operation and w. certified instructors UAS Specific operations risk assessment competency by EASA / ICAO (2 days, up to 15 hours at location of training provider, for up to 4 trainees) Drone Aircraft system specs • Weight, with batteries max 9780g • Max takeoff weight: minimum 15.8 kg • Max Payload: minimum 6 kg • Diagonal Wheelbase, min 1070 mm • Dimensions: max 980×760×480 mm unfolded, • Carrying case dimensions: max 779×363×528 mm • Max Horizontal Speed (at sea level, no wind) min 25 m/s, Max Ascent Speed min 10 m/s, Max Descent Speed min 8 m/s • Max Flight Time (no wind) min 59 minutes • Max Hover Time (no wind) min. 53 minutes • Max Wind Speed Resistance min 12 m/s • Operating Temperature range min -20° to 50° C • Equipped with airborne ADS-B In receiver and dual antennas, supporting reception up to min 20 km. • Supported GNSS systems: min GPS + Galileo + BeiDou + GLONASS • Hovering Accuracy (with RTK positioning) minimum Horizontal: ±10 cm, Vertical: ±10 cm • RTK/GNSS System: Antenna number min 2, • RTK GNSS Accuracy in RTK Fix: min 1 cm + 1 ppm (horizontal), 1.5 cm + 1 ppm (vertical) • RTK Heading accuracy better than 2° • IP Rating min IP55 • Supported number of simultaneous gimbals and payloads: minimum 3 • Drone Sensing includes minimum: Omnidirectional binocular vision system (surround view provided by full-color fisheye vision sensors), Horizontal rotating LiDAR, upper LiDAR, and downward 3D infrared range sensor and Six-direction mmWave radar • Drone Sensing Omnidirectional FOV: minimum 90° (horizontal), 90° (vertical) in all directions w. detection range at least 1-100m and measurement range at least 0.6-21m • Drone Sensing Rotating LiDAR FOV: min 360° (horizontal), 58° (vertical), w measurement range for Power Lines at least 35 m @ 30° @ 10,000 lux for 21.6 mm steel-core aluminum stranded wire • Drone Sensing mmWave Radar FOV: min ± 45° (horizontal and vertical), w. measurement range for Power Lines at least 36 m for a 12mm steel-core aluminum stranded wire • FPV Camera framerate min 30fps w. resolution min 1080p, and DFOV min 150° • Video Transmission Type: O4 Enterprise Enhanced Video Transmission System • Operating Frequency and Transmitter Power (EIRP) 2.4000-2.4835 GHz < 20 dBm (CE), 5.150-5.250 GH < 23 dBm (CE), 5.725-5.850 GHz < 14 dBm (CE) • Max Transmission Distance (unobstructed) min 20 km (CE) • Antenna Operating Mode: 2T4R • Supports Dual Control Mode • Drone Remote Controller: Screen min 7-inch LCD touchscreen, with a resolution of min 1920×1200 pixels, and high brightness of min 1400 cd/m2, Builtin battery min 46.9Wh w. runtime min 3.8h, ,Charging type: USB-C, Charging time: max 2 hours for internal battery, • Includes auxiliary RC battery w. energy min 37Wh and runtime min 3.2h • Includes RC shoulder strap and holding bracket Intelligent Flight Battery spec • Model: TB100 or equivalent, Weight: max 4740g, • Nominal Voltage: Min 48V (13S), Capacity: Min 20254mAh (977 Wh) • Quantity: 3pcs Intelligent Battery Station spec • Model: BS100 or equivalent approved by Drone and Battery manufacturer, • Output Channels min: 3x TB100 and 2x WB37 • Charging Time for TB100 from 0% to 100% at 220 V supply: no more than 45 minutes in Fast Charging, no more than 110 minutes in Silent Mode Hyperspectral/multispectral imaging camera specs • Model: FS50/90 or equivalent, compatible with Drone, Shooting method: Real-time acquisition • Spatial Resolution: min 1920 (2K), Spectral Resouliton/Half-wave Width: max 3.5nm • Spectral Range min 400-1000nm. Number of Spectral channels min 90 • Image Sensor: not smaller than 1/1.1 inch CMOS, Effective pixels min 1920, Shutter Type: Global shutter, Sampling rate min 128 line/s, Quantization: min 12bit • Optical window type: high transmission optical glass • FOV min 25°, Ground resolution from 120m altitude at least 2.8cm or better, Coverage from 120m alt. 54m or better • Weight: no more than 840g • Power consumption max 45W, Power supply connector: X-Port • Image format: 12bit.SPE (compatible with third party analysis software such as envi) • Ground station real-time preview data acquisition • Data storage: at least 512SSD • Reflectance calibration target of 80% reflectivity, min size 50cm*50cm, Quantity: 1 • Software license for UAV real-time flight control software, Merge puzzle software, and image analysis software, |  |  |  |
| 2 | Drone system for a fertilizer and plant protection application Quantity: 1 Unit set (Unmanned treatment system specialised for forestry) |  |  |  |
|  | Required contents and characteristics of the Unit set: • Drone Aircraft system, capable of running accompanied precise airflow-supported spraying module and precise granule spreading module, with Remote Controls and standard accessories, incl. Cellular Dongle for RC, auxiliary RC batteries and suitable RC charger and hub • Drone Spraying module w. liquids tank, precise pumps, flowmeters, equipped w. Drone Spraying System extension to provide total of 4 centrifugal dual-motor Mist nozzles • Drone Spreading module, w. materials tank, feeder screws and spreading disc device • Standard manufacturers approved flight battery set for drone (3 intelligent flight batteries) • Manufacturers approved Multifunctional Inverter Generator w. integrated fast charger for flight batteries and auxiliary power output • Relay signal transmission system for Drone • RTK base station for precise positioning of Drone with tripod and standard accessories • Full set of spare propellers for Drone • Auxiliary mapping system for mission paths preparation in forestry, nurseryes and orchard and for application map preparation DJI Mavic 3M with Enterprise Battery kit • Certified installation of the system and technical support by official manufactures maintenance provider with at least 2 certified maintenance engineers • Certified training in the automated application of the system for herbal treatment in forestry, nurseries and orchards, provided by at least 2 trainers w. official instructors certification of the Drone manufacturers Academy and w. official aviation instructors competency by EASA / ICAO in domain of professional UAS operation and w. certified instructors UAS Specific operations risk assessment competency by EASA / ICAO (3 days, up to 25 hours at location of training provider, for up to 4 trainees) Drone Aircraft system specs • Weight, when equipped for spraying with 4 nozzles: max 77 kg (incl. flight battery) • Max takeoff weight for spraying when equipped for spraying with 4 nozzles: 177 kg • Max Diagonal Wheelbase 2330 mm • Dimensions: max 3220 × 3224 × 975 mm (arms and propellers unfolded), max 1105 × 1265 × 975 mm (arms and propellers folded) • Hovering Accuracy (with strong GNSS signal) RTK enabled: Horizontal: ±10 cm, Vertical: ±10 cm RTK disabled: Horizontal: ±60 cm, Vertical: ±30 cm • RTK/GNSS System: Antenna number min 2, • Max Configurable Flight Radius 2000 m • Max Wind Resistance min 6 m/s • Max Takeoff Altitude min 4500m • IP Rating min IP67 • Drone Propulsion System Motors: Stator Size min 155×16 mm, KV 60 rpm/V, Number of motors: Min 8 • Drone Propulsion System Propellers: Carbon Fiber Composite, Dimensions 62", Number: 8 • Drone Safety System includes minimum: Laser obstacle sensing (min FOV Vertical 272° Horizontal 60°), Front radar (min FOV Horizontal 360°, Vertical ± 45°), Downward Radar (min FOV Left to Right ± 12.5°, Front to Back ± 22.5°), Rear Radar (min FOV Left to Right ± 45°, Front and Back ± 45°), Quad vision system (min FOV Horizontal 360°, Vertical 180°), Millimetre wave radar EIRP max < 20 dBm, Measurement range minimum to 60 m, Effective safe obstacle avoidance speed minimum to 13.8 m/s, Safety limit distance: max 2.5 m (distance between the drone and obstacle after braking and stable hovering) • Drone Remote Controller: Operating Frequency and EIRP: 2.4000-2.4835 GHz <20 dBm, 5.725-5.850 GHz <14 dBm, Screen: Min 7-inch LCD touchscreen, with a resolution of Min 1920×1200 pixels, and high brightness of Min 1400 cd/m2, Internal Battery Runtime: Min 3.8 hours, External Battery Runtime: Min 3.2 hours, Charging type: USB-C, Charging time: max 2 hours for internal battery, Includes RC Cellular Dongle for mobile internet access, • Includes auxiliary RC battery w. energy min 37Wh, type 2S, Quantity min 2 pcs, Includes auxiliary RC battery charging hub and RC USB-C charger w. min power output 65W Drone Spraying System module specs • Dimensions max 880 × 870 × 810 mm (spray rod folded), Spray Tank: Material: Plastic (HDPE), Volume: Min 100 L, Operating Payload: Min 100 kg, Delivery pumps quantity 2, Pump type: Impeller pump w. Magnetic drive, Pump flow rate min 40 L/min w. 4 nozzle system, equipped w. 4 centrifugal mist nozzles LX09550SX, Nozzle distance outermost 1830-1840 mm, Droplet size range min 50-500 μm, Effective spray width in range min 5-13m, Quantity: 1 Drone Spreading System module specs • Materials tank volume min 150 L, Maximum operational load min 100kg, Discharge type: screw feeder feeding, Spreading type: centrifugal disc, Discharge rate min 400 kg/min (for compound fertilizer), Effective spreading width range min 3-10m, Quantity: 1 Intelligent Flight Battery spec • Model: DB2160 or equivalent, Weight: max 15 kg, • Capacity: Min 41000 mAh, Nominal Voltage: Min 52V, Quantity: 3pcs Multifunctional Inverter Generator w. integrated Battery Charger specs • Model: D14000iE or equivalent, approved by Drone and Battery manufacturer, Output Channels min: Battery DC recharge (42 - 61.6 V/11500 W), Air-cooled radiator charge (12 V/6 A), Auxiliary AC output (220 V/1500 W), Net weight max 88 kg, • Fuel type: Unleaded gasoline with RON ≥91, Fuel tank capacity min 30 L • Starting Method: electric start via the One-Button Start Switch, powered from Drone Battery • Output Battery Charging Time: max 9 minutes (DB2160 Intelligent Flight Battery, 30% - 95% charge, optimal conditions) Relay signal transmission system specs • Signal relay system, compatible w. Drone and RC, model O4 Relay or equivalent • Internal battery Capacity min 6500 mAh, Operating Time min 5 hours, Charging Time: max 2.5 hours, Chagring prort: USB-C type • Ingress protection min IP55 • Operating Frequency 2.4000 - 2.4835 GHz, 5.150 - 5.250 GHz, 5.725 - 5.850 GHz, Effective signal range min 4 km (CE, in an interference-free, unobstructed environment, at flight altitude 2.5 m) RTK base station specs • D-RTK3 Multifunctional base station or equivalent, compatible with drones RTK system, with tripod and positioning pole • Internal battery Capacity min 6500 mAh, Operating Time min 7 hours, Charging Time: max 2.5 hours, Chagring prort: USB-C type • Operating Frequency Broadcast Mode: O4 2.4G/5.8G • Simultaneously receive GPS: L1/L2/L5, BeiDou: B1I/B2I/B3I/B1C/B2A/B2B, Galileo: E1/E5a/E5b/E6 GLONASS: G1/G2, QZSS: L1CA/L2C/L5, L-BAND • Positioning Accuracy after calibration: Horizontal: 0.8 cm + 1 ppm (RMS), Vertical: 1.5 cm + 1 ppm (RMS) • IP Rating: min IP67 Full set of spare propellers for Drone • Manufacturers approved CW propellers (propeller blade pair) for Drone, Quantity: 4psc • Manufacturers approved CCW propellers (propeller blade pair) for Drone, Quantity: 4psc |  |  |  |

| All items | Delivery of all items to following address: “Forest Administration” SREMSKA MITROVICA, Parobrodska 2, 22000 Sremska Mitrovica, Republic of Serbia. |  |  |  |
| --- | --- | --- | --- | --- |
| All items | Technical documentation for equipment as per manufacturers standard (Operating manuals/ Users Guide/ Equipment operating instructions/ Cleaning procedures/ Maintenance procedures/ Calibration procedures) upon delivery. |  |  |  |
| All items | Training minimum one day training by 2 instructors rated trainers by Drone manufacturers Academy and w. official aviation instructor competency by EASA / ICAO in domain of professional UAS operation and w. certified instructors' competency in UAS Specific operations risk assessment by EASA / ICAO |  |  |  |
| All items | Warranty Warranty period of minimum 365 days from the issuance of PAC (Provisional Acceptance Certificate) Offer must include service specificaition including: - Guaranteed that any requests for services will be responded within 48 hours. - Guarantee that all items can be repaired or alternatively replaced as per manufacturers warranty and maintenance terms and conditions. - Letter of confirmation that genuine spare parts and consumables will be available for a period of minimum 3 years from the date of final acceptance of the equipment. |  |  |  |
| All items | Commercial warranty 365 days from the issuance of FAC (Final Acceptance Certificate) in accordance with the conditions laid down in Article 32 of the General Conditions and Article 33 of the Special Conditions. Detailed description of the organisation of the proposed service and description of the Manufacturer’s commercial warranty shall be included in the offer. |  |  |  |

<Signature of authorised representative of the legal entity >

< Name and position of authorised representative of the legal entity >

< Date >