# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Public procurement of supplies for project Siga2Kazuk

Ref. Number: HR-RS00272 – LP – TD02

Lot 1: Drone with RGB and Lidar sensor

Columns 1-2 should be completed by the Project partner

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Project partner shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

The requirements set out in the technical specifications represent the minimum technical characteristics which offered goods must satisfy, unless stated otherwise, and tenderers are not allowed to modify technical specification in any way.

For each item for which it is not explicitly stated that it is allowed to offer goods of the equal characteristics, i.e. for each item where it is not stated “or equivalent”, for the purposes of this tender documentation it is assumed that words “or equivalent” are stated, and tenderer is allowed to offer equivalent goods / goods of equivalent characteristics.

General Requirements

•	All requirements stated and outlined in this document must be regarded as mandatory and the minimum acceptable criteria. All requirements outlined in this document are accompanied by the phrase "or equivalent".

•	The tenderer is required to provide the specifications of the offered items in the Technical Offer, including details such as the manufacturer, product type, model, and country of origin, and all the other relevant documents as per specification required. All documentation must be provided in English or Serbian, both in hardcopy and electronic formats.

•	The selected contractor is required to provide all necessary documentation for the registration of the completed vehicle in accordance with the relevant legal regulations in the Republic of Serbia.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s decision (Y/N) |
| --- | --- | --- | --- | --- |
| 1 | Drone with RGB and Lidar sensor Quantity: 1 Unit set (Unmanned treatment system specialised for forestry) |  |  |  |
|  | Required contents and characteristics of the Unit set Platform: - Type of drone: Professional VTOL fixed-wing unmanned aerial drone. Drone for vertical take-off, landing and stationary maintenance in the air. The drone is equipped with sensors for monitoring the complete state of the system and built-in intelligent systems in the event of a safe emergency landing. The body of the drone is made of fiberglass, so it gives stability and aerodynamics to the drone during flight. - Data Acquisition Sensors: RGB and Lidar sensor, modular, easily replaceable (according to mission requirements) without the use of tools - Purpose of the drone: Photogrammetry, LiDAR mapping and data acquisition for digital modeling Wingspan: 125cm - Radio link: Two-way range 10 km - Maximum weight at take-off: 5.2 kg or more - Load capacity: 1250 g - Maximum flight time with RGB camera: Minimum 59 minutes at an altitude from 0m to 500m - Maximum flight time with LiDAR sensor: Minimum 45 minutes at an altitude from 0m to 500m - Operating speed: Adaptive, at least 16–22 m/s - Wind resistance: 12 m/s - Gusts of wind: 18 m/s - Maximum working height: Up to 5,000 m AMSL - Operating Temperature: min. range -10 to 40⁰C - Degree of protection: IP53 - Automatic take-off and landing: Mandatory - Accuracy of automatic take-off and landing: min. 2m - Fully autonomous mission execution: Mandatory - Automatic return to the place of departure: Mandatory - GNSS: Multi-frequency GNSS with PPK support Navigation and positioning - PPK georeferencing: Mandatory - Supported GNSS systems: GPS, GLONASS, Galileo and BeiDou - Absolute accuracy (RMS x,y,z): Up to 3 cm RMS with PPK processing RGB sensor - General: Easy and simple mounting and dismounting of the sensor - Resolution: Minimum of 61 MP, low distortion - Weight with bracket: 440g - Sensor Type: Full Frame - Lens: Approx. 17 mm - Minimum GSD: Minimum 1.2 cm/pixel - Coverage: @120m 550 ha per flight, GSD: 2.7cm - Front camera skew: Yes 15° - The relative accuracy of the length: 0.005% - Minimum shutter speed: 0.4s - Automatic geotagging: Mandatory Lidar sensor - Sensor Type: Professional UAV LiDAR - Point density at 45m, one pass, one return of signal 50% side overlap: 110 points/m2 - Coverage: @90m 460 ha per flight, with 30% side overlap - Vertical Absolute Accuracy: 3cm - Field of view – horizontal: 90⁰ - Field of view – vertical: 40.3⁰ - Number of returns: 3 - Range: 0.5 to 300m, 80m on a surface with 10% reflectivity - Integrated IMU: Mandatory, pitch/roll accuracy 0.006°, direction accuracy 0.03° - Integrated GNSS: Mandatory, integrated on LIDAR, constellation: GPS, GLONASS; BEIDOU; GALILEO - PPK georeferencing: Mandatory - Automatic synchronization: Mandatory - Output Format: LAS & LAZ - Georeferenced point cloud: Mandatory - Note: The RGB sensor is not an integral part of the LiDAR sensor and it should be supplied as a separate sensor Flight planning software - General: Same manufacturer as the drone, android oriented or available on portable devices - Mission planning: Autonomous planning of photogrammetric and LiDAR missions - Adaptation to the terrain: DTM planning and automatic altitude maintenance. Pause and resume mission in the field. Manual control of the system on landing if necessary. Corridor mapping option. KML loading - Optional: The ability to use cloud services to plan and track flight execution Safety systems - Return to home: Mandatory - Automatic landing at low battery: Mandatory - System monitoring: Continuous monitoring of the drive, battery and navigation - Avoidance of static obstacles: Optional, upgradable, detection technology: Radar with a larger number of antennas, range 100m - Obstacle Warning: Mandatory - Automatic parachute: Optional, the ability to install an automatic parachute, for safe flight over populated areas. In addition to the automatic parachute, the parachute can also be activated manually with a special device. Minimum height of parachute activation 45m. - Flight logs: Mandatory Scope of delivery Additional equipment: Transport case Three additional sets of batteries Field charging equipment Spare landing gear (fin and tail) Spare propellers Software for RGB sensor images processing with the following characteristics - Photogrammetric triangulation - Generating and editing dense point clouds - Generation and texture of 3D models - Creating spherical panoramic photos - Support for wide-angle cameras (fisheye) - Support for spherical and cylindrical camera types - Classification of dense point cloud - Creating and exporting DEM - Creating a georeferenced Ortho mosaic - Editing the Ortho mosaic of the seamline - Support for markers and scales - Support for checkpoints - Automatic recognition of coded and unencoded markers - Processing of multispectral images - Generation and calculation of vegetation indices - 4D modelling for dynamic scenes - Python coding - Online processing -OS: Windows, Mac OS X, Debian/Ubuntu |  |  |  |

| All items | Delivery of all items to following address: “Forest Estate” Sombor, Apatinski put 11, Sombor, Republic of Serbia. |  |  |  |
| --- | --- | --- | --- | --- |
| All items | Installation and testing of all functions of the instrument, to manufacturers standard on samples commonly used for the corresponding instrument. Installed equipment must be tested as system, compatible with existing system. |  |  |  |
| All items | Technical documentation for equipment as per manufacturers standard (Operating manuals/ Users Guide/ Equipment operating instructions/ Cleaning procedures/ Maintenance procedures/ Calibration procedures) upon delivery. |  |  |  |
| All items | - Training minimum day event/training for minimum 5 Contracting Authority employees. |  |  |  |
| All items | Warranty Warranty period 365 days from the issuance of PAC (Provisional Acceptance Certificate) in accordance with article 32 SC and GC of the Contract. Offer must include warranty service description including: - Service organisation contact data including name, postal address, telephone number, fax number and e-mail address; - Guaranteed that any requests for services will be attended to within 48 hours; - Guarantee that all items can be repaired or alternatively replaced within a maximum of 7 days; Letter of confirmation that genuine spare parts and consumables will be available for a period of minimum 3 years from the date of final acceptance of the equipment. |  |  |  |
| All items | Commercial warranty 365 days from the issuance of FAC (Final Acceptance Certificate) in accordance with the conditions laid down in Article 32 of the General Conditions and Article 33 of the Special Conditions. Detailed description of the organisation of the proposed service and description of the Manufacturer’s commercial warranty shall be included in the offer. |  |  |  |

<Signature of authorised representative of the legal entity >

< Name and position of authorised representative of the legal entity >

< Date >