# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Public procurement of supplies for project Siga2Kazuk

Ref. Number: HR-RS00272 – LP – TD02

Lot 2: Spraying Drone for Applying Plant Nutrition Products

Columns 1-2 should be completed by the Project partner

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Project partner shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

The requirements set out in the technical specifications represent the minimum technical characteristics which offered goods must satisfy, unless stated otherwise, and tenderers are not allowed to modify technical specification in any way.

For each item for which it is not explicitly stated that it is allowed to offer goods of the equal characteristics, i.e. for each item where it is not stated “or equivalent”, for the purposes of this tender documentation it is assumed that words “or equivalent” are stated, and tenderer is allowed to offer equivalent goods / goods of equivalent characteristics.

General Requirements

•	All requirements stated and outlined in this document must be regarded as mandatory and the minimum acceptable criteria. All requirements outlined in this document are accompanied by the phrase "or equivalent".

•	The tenderer is required to provide the specifications of the offered items in the Technical Offer, including details such as the manufacturer, product type, model, and country of origin, and all the other relevant documents as per specification required. All documentation must be provided in English or Serbian, both in hardcopy and electronic formats.

•	The selected contractor is required to provide all necessary documentation for the registration of the completed vehicle in accordance with the relevant legal regulations in the Republic of Serbia.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s decision (Y/N) |
| --- | --- | --- | --- | --- |
| 1 | Spraying Drone for Applying Plant Nutrition Products Quantity: 1 Unit set (Unmanned treatment system specialised for forestry) |  |  |  |
|  | Required contents and characteristics of the Unit set: • Drone Aircraft system, capable of running accompanied precise airflow-supported spraying module and precise granule spreading module, with Remote Controls and standard accessories, incl. Cellular Dongle for RC, auxiliary RC batteries and suitable RC charger and hub • Drone Spraying module w. liquids tank, precise pumps, flowmeters, equipped w. Drone Spraying System extension to provide total of 4 centrifugal dual-motor Mist nozzles • Drone Spreading module, w. materials tank, feeder screws and spreading disc device • Standard manufacturers approved flight battery set for drone (3 intelligent flight batteries) • Manufacturers approved Multifunctional Inverter Generator w. integrated fast charger for flight batteries and auxiliary power output • Relay signal transmission system for Drone • RTK base station for precise positioning of Drone with tripod and standard accessories • Full set of spare propellers for Drone • Auxiliary mapping system for mission paths preparation in forestry, nurseryes and orchard and for application map preparation DJI Mavic 3M with Enterprise Battery kit • Certified installation of the system and technical support by official manufactures maintenance provider with at least 2 certified maintenance engineers • Certified training in the automated application of the system for herbal treatment in forestry, nurseries and orchards, provided by at least 2 trainers w. official instructors certification of the Drone manufacturers Academy and w. official aviation instructors competency by EASA / ICAO in domain of professional UAS operation and w. certified instructors UAS Specific operations risk assessment competency by EASA / ICAO (3 days, up to 25 hours at location of training provider, for up to 4 trainees) Drone Aircraft system specs • Weight, when equipped for spraying with 4 nozzles: max 77 kg (incl. flight battery) • Max takeoff weight for spraying when equipped for spraying with 4 nozzles: 177 kg • Max Diagonal Wheelbase 2330 mm • Dimensions: max 3220 × 3224 × 975 mm (arms and propellers unfolded), max 1105 × 1265 × 975 mm (arms and propellers folded) • Hovering Accuracy (with strong GNSS signal) RTK enabled: Horizontal: ±10 cm, Vertical: ±10 cm RTK disabled: Horizontal: ±60 cm, Vertical: ±30 cm • RTK/GNSS System: Antenna number min 2, • Max Configurable Flight Radius 2000 m • Max Wind Resistance min 6 m/s • Max Takeoff Altitude min 4500m • IP Rating min IP67 • Drone Propulsion System Motors: Stator Size min 155×16 mm, KV 60 rpm/V, Number of motors: Min 8 • Drone Propulsion System Propellers: Carbon Fiber Composite, Dimensions 62", Number: 8 • Drone Safety System includes minimum: Laser obstacle sensing (min FOV Vertical 272° Horizontal 60°), Front radar (min FOV Horizontal 360°, Vertical ± 45°), Downward Radar (min FOV Left to Right ± 12.5°, Front to Back ± 22.5°), Rear Radar (min FOV Left to Right ± 45°, Front and Back ± 45°), Quad vision system (min FOV Horizontal 360°, Vertical 180°), Millimetre wave radar EIRP max < 20 dBm, Measurement range minimum to 60 m, Effective safe obstacle avoidance speed minimum to 13.8 m/s, Safety limit distance: max 2.5 m (distance between the drone and obstacle after braking and stable hovering) • Drone Remote Controller: Operating Frequency and EIRP: 2.4000-2.4835 GHz <20 dBm, 5.725-5.850 GHz <14 dBm, Screen: Min 7-inch LCD touchscreen, with a resolution of Min 1920×1200 pixels, and high brightness of Min 1400 cd/m2, Internal Battery Runtime: Min 3.8 hours, External Battery Runtime: Min 3.2 hours, Charging type: USB-C, Charging time: max 2 hours for internal battery, Includes RC Cellular Dongle for mobile internet access, • Includes auxiliary RC battery w. energy min 37Wh, type 2S, Quantity min 2 pcs, Includes auxiliary RC battery charging hub and RC USB-C charger w. min power output 65W Drone Spraying System module specs • Dimensions max 880 × 870 × 810 mm (spray rod folded), Spray Tank: Material: Plastic (HDPE), Volume: Min 100 L, Operating Payload: Min 100 kg, Delivery pumps quantity 2, Pump type: Impeller pump w. Magnetic drive, Pump flow rate min 40 L/min w. 4 nozzle system, equipped w. 4 centrifugal mist nozzles LX09550SX, Nozzle distance outermost 1830-1840 mm, Droplet size range min 50-500 μm, Effective spray width in range min 5-13m, Quantity: 1 Drone Spreading System module specs • Materials tank volume min 150 L, Maximum operational load min 100kg, Discharge type: screw feeder feeding, Spreading type: centrifugal disc, Discharge rate min 400 kg/min (for compound fertilizer), Effective spreading width range min 3-10m, Quantity: 1 Intelligent Flight Battery spec • Model: DB2160 or equivalent, Weight: max 15 kg, • Capacity: Min 41000 mAh, Nominal Voltage: Min 52V, Quantity: 3pcs Multifunctional Inverter Generator w. integrated Battery Charger specs • Model: D14000iE or equivalent, approved by Drone and Battery manufacturer, Output Channels min: Battery DC recharge (42 - 61.6 V/11500 W), Air-cooled radiator charge (12 V/6 A), Auxiliary AC output (220 V/1500 W), Net weight max 88 kg, • Fuel type: Unleaded gasoline with RON ≥91, Fuel tank capacity min 30 L • Starting Method: electric start via the One-Button Start Switch, powered from Drone Battery • Output Battery Charging Time: max 9 minutes (DB2160 Intelligent Flight Battery, 30% - 95% charge, optimal conditions) Relay signal transmission system specs • Signal relay system, compatible w. Drone and RC, model O4 Relay or equivalent • Internal battery Capacity min 6500 mAh, Operating Time min 5 hours, Charging Time: max 2.5 hours, Chagring prort: USB-C type • Ingress protection min IP55 • Operating Frequency 2.4000 - 2.4835 GHz, 5.150 - 5.250 GHz, 5.725 - 5.850 GHz, Effective signal range min 4 km (CE, in an interference-free, unobstructed environment, at flight altitude 2.5 m) RTK base station specs • D-RTK3 Multifunctional base station or equivalent, compatible with drones RTK system, with tripod and positioning pole • Internal battery Capacity min 6500 mAh, Operating Time min 7 hours, Charging Time: max 2.5 hours, Chagring prort: USB-C type • Operating Frequency Broadcast Mode: O4 2.4G/5.8G • Simultaneously receive GPS: L1/L2/L5, BeiDou: B1I/B2I/B3I/B1C/B2A/B2B, Galileo: E1/E5a/E5b/E6 GLONASS: G1/G2, QZSS: L1CA/L2C/L5, L-BAND • Positioning Accuracy after calibration: Horizontal: 0.8 cm + 1 ppm (RMS), Vertical: 1.5 cm + 1 ppm (RMS) • IP Rating: min IP67 Full set of spare propellers for Drone • Manufacturers approved CW propellers (propeller blade pair) for Drone, Quantity: 4psc • Manufacturers approved CCW propellers (propeller blade pair) for Drone, Quantity: 4psc |  |  |  |

| All items | Delivery of all items to following address: “Forest Management Unit” Monoštorske šume, Department 48, Parobrodska 2, oslobođenja 64, Bački Monoštor (on the terrain), Republic of Serbia. |  |  |  |
| --- | --- | --- | --- | --- |
| All items | Installation and testing of all functions of the instrument, to manufacturers standard on samples commonly used for the corresponding instrument. Installed equipment must be tested as system, compatible with existing system. |  |  |  |
| All items | Technical documentation for equipment as per manufacturers standard (Operating manuals/ Users Guide/ Equipment operating instructions/ Cleaning procedures/ Maintenance procedures/ Calibration procedures) upon delivery. |  |  |  |
| All items | Training minimum day event/training for minimum 5 Contracting Authority employees. |  |  |  |
| All items | Warranty Warranty period 365 days from the issuance of PAC (Provisional Acceptance Certificate) in accordance with article 32 SC and GC of the Contract. Offer must include warranty service description including: - Service organisation contact data including name, postal address, telephone number, fax number and e-mail address; - Guaranteed that any requests for services will be attended to within 48 hours; - Guarantee that all items can be repaired or alternatively replaced within a maximum of 7 days; - Letter of confirmation that genuine spare parts and consumables will be available for a period of minimum 3 years from the date of final acceptance of the equipment. |  |  |  |
| All items | Commercial warranty 365 days from the issuance of FAC (Final Acceptance Certificate) in accordance with the conditions laid down in Article 32 of the General Conditions and Article 33 of the Special Conditions. Detailed description of the organisation of the proposed service and description of the Manufacturer’s commercial warranty shall be included in the offer. |  |  |  |

<Signature of authorised representative of the legal entity >

< Name and position of authorised representative of the legal entity >

< Date >