# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Public procurement of supplies for project Siga2Kazuk

Ref. Number: HR-RS00272 – LP – TD02

Lot 3: The Solar-Powered Irrigation System

Columns 1-2 should be completed by the Project partner

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Project partner shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

The requirements set out in the technical specifications represent the minimum technical characteristics which offered goods must satisfy, unless stated otherwise, and tenderers are not allowed to modify technical specification in any way.

For each item for which it is not explicitly stated that it is allowed to offer goods of the equal characteristics, i.e. for each item where it is not stated “or equivalent”, for the purposes of this tender documentation it is assumed that words “or equivalent” are stated, and tenderer is allowed to offer equivalent goods / goods of equivalent characteristics.

General Requirements

•	All requirements stated and outlined in this document must be regarded as mandatory and the minimum acceptable criteria. All requirements outlined in this document are accompanied by the phrase "or equivalent".

•	The tenderer is required to provide the specifications of the offered items in the Technical Offer, including details such as the manufacturer, product type, model, and country of origin, and all the other relevant documents as per specification required. All documentation must be provided in English or Serbian, both in hardcopy and electronic formats.

•	The selected contractor is required to provide all necessary documentation for the registration of the completed vehicle in accordance with the relevant legal regulations in the Republic of Serbia.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s decision (Y/N) |
| --- | --- | --- | --- | --- |
| 1 | Spraying The Solar-Powered Irrigation System Quantity: Complete (Unmanned treatment system specialised for forestry) |  |  |  |
|  | Required contents and characteristics: Solar Plant - Solar plant for the production of electricity on the ground with a total power of 15kW -16.5kw of installed power of panels with galvanized construction - monocrystalline solar panels with a power of 540W. Pump - Suction basket Ø75mm with flap and supporting material for connection - Suction pipe Ø75mm L=6m with supporting material for connection - Horizontal electric pump with operating points: Q=25m3/h P=35m Q=15m3/h P=40m and a control cabinet for power supply from the solar plant. The control cabinet supports manual and automatic operation, contains a frequency inverter 11kW, pressure transmitter, dry-running protection and a 230V power source. - Solar pump system with inverter from DC to AC Filter Station - Pressure pipeline Ø75mm made of PVC-U material with supporting elements (check valve, service valves, air vent) - Automatic disk filter Q=15-30m3/h, P=8bar - Water meter DN 50 Qmin=3m3/h Qmax=70m3/h with pulse transducer - Fertigation system with Venturi pipe Q=150l/h, feeding tank with lid V=1000 l and chemically resistant filter, supporting equipment for connecting the tank and the feeding system Distribution Pipeline - Lay flat pipeline with a diameter of 3"/4bar, UV resistant quantity: 1,500m - Lay flat pipeline with a diameter of 2"/4bar, UV resistant quantity: 800m - Solenoid valve with pressure regulation 0-4bar and three-arm valve Open-Auto-Close, 2"UN. quantity: 8 pieces - Trailing couplings for connecting distribution pipeline and control valve – set Drip tapes - Drip tape 150 mic with integrated dropper with labyrinth, flow rate 0.65 l/h, distance between droppers 0.3 m with accompanying equipment for connection to the distribution pipeline, quantity: 120,000 m. |  |  |  |

| All items | Delivery of all items to following address: “Forest Management Unit” Monoštorske šume, Department 48, Parobrodska 2, oslobođenja 64, Bački Monoštor, Republic of Serbia. |  |  |  |
| --- | --- | --- | --- | --- |
| All items | Installation and testing of all functions of the instrument, to manufacturers standard on samples commonly used for the corresponding instrument. Installed equipment must be tested as system, compatible with existing system. |  |  |  |
| All items | Technical documentation for equipment as per manufacturers standard (Operating manuals/ Users Guide/ Equipment operating instructions/ Cleaning procedures/ Maintenance procedures/ Calibration procedures) upon delivery. |  |  |  |
| All items | Warranty Warranty period 365 days from the issuance of PAC (Provisional Acceptance Certificate) in accordance with article 32 SC and GC of the Contract. Offer must include warranty service description including: - Service organisation contact data including name, postal address, telephone number, fax number and e-mail address; - Guaranteed that any requests for services will be attended to within 48 hours; - Guarantee that all items can be repaired or alternatively replaced within a maximum of 7 days; - Letter of confirmation that genuine spare parts and consumables will be available for a period of minimum 3 years from the date of final acceptance of the equipment. |  |  |  |
| All items | Commercial warranty 365 days from the issuance of FAC (Final Acceptance Certificate) in accordance with the conditions laid down in Article 32 of the General Conditions and Article 33 of the Special Conditions. Detailed description of the organisation of the proposed service and description of the Manufacturer’s commercial warranty shall be included in the offer. |  |  |  |

<Signature of authorised representative of the legal entity >

< Name and position of authorised representative of the legal entity >

< Date >