# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supplementary fire truck and firefighting equipment and tools for project STOP FIRES	p 1 /…

Publication reference: TD05 - HUSRB/23R/11/086–5.1.4

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

General Requirements

•	All requirements stated and outlined in this document must be regarded as mandatory and the minimum acceptable criteria. All requirements outlined in this document are accompanied by the phrase "or equivalent".

•	The tenderer is required to provide the specifications of the offered items in the Technical Offer, including details such as the manufacturer, product type, model, and country of origin. All documentation must be provided in English or Serbian, both in hardcopy and electronic formats.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Fire Extinguishing Tank, Quantity: 1 Unit |  |  |  |
|  | Required technical characteristics: 6.000 l, Galvanized Fire Extinguishing Tank With Air Brakes - Volume: min. 6.000 l - Width: max. 2.18m - Length: max. 6.40m - Height: max. 2.70m - Tires: min. 16.0/70*20" - Working Pressure: min. 1.00bar - Vacuum: 90% - Pump Output: min. 5.700 l/min - Towing Needs: min. 50kw - Braking system: pneumatic |  |  |  |
| 2 | Electric motor for electric winding Quantity: 1 Unit |  |  |  |
|  | Required technical characteristics: - Performance Range: 55 l/min at 40 bar to 135 l/min at 20 bar (diaphragm) or 42 l/min at 100 bar to 21 l/min at 250 bar (piston). - Engine Types: min. 9, HP Petrol; or 10 HP Diesel. |  |  |  |
| 3 | Nets for front and rear lights Quantity: 1 Unit |  |  |  |
|  | Required technical characteristics: Protection Nets for front and rear lights for Ford Ranger |  |  |  |
| 4 | Floodlight Quantity: 1 Unit |  |  |  |
|  | Required technical characteristics: - Voltage: min. 10-30V - Ampere: min. 3A - Power: min. 28W - Brightness: min. 2.700Lm - Water resistance: min. IP65 - Beam Distance: min. 300-400m - Rotation: min. 360 degrees horizontally and 60 degrees vertically - Included: remote control |  |  |  |
| 5 | Cargo cover for the vehicle Quantity: 1 Unit |  |  |  |
|  | Required technical characteristics: - Type: hardtop pop-up with side door |  |  |  |
| 6 | Chainsaw – Type 1 Quantity: 1 Unit |  |  |  |
|  | Required technical characteristics: - Volume: min. 35.8 cm³: - Power: min. 1.60 kW ; min. 2.20 PS - Unit weight without fuel: max. 4.5 kg - The weight of the system without fuel: max. 5.18 kg - Strength-to-weight ratio: min. 2.80 kg/kW - Bar length: min. 35 cm - Groove width: min. 1.30 mm - The length of the device with a stop: min. 400 mm - Vibration value left: max. 2.5 m/s² - Vibration value on the right: 3.4 m/s² |  |  |  |
| 7 | Chainsaw – Type 2 Quantity: 2 Units |  |  |  |
|  | Required technical characteristics: - Volume: min. 59 cm³: min. 1.60 kW - Power: min. 3.50 kW ; min. 4.80 PS - Unit weight without fuel: max. 5.6 kg - The weight of the system without fuel: max. 6.70 kg - Strength-to-weight ratio: min. 1.60 kg/kW - Bar length: min. 40 cm - Groove width: min. 1.60 mm - The length of the device with a stop: min. 460 mm - Vibration value left: max. 3.5 m/s² - Vibration value on the right: 3.5 m/s² |  |  |  |

| All items | Delivery of all items to following address: Graničarska BB, 24000 Subotica, Republic of Serbia. |  |  |  |
| --- | --- | --- | --- | --- |
| All items | Installation and testing of all functions of the instrument, to manufacturers standard on samples commonly used for the corresponding instrument. Installed equipment must be tested as system, compatible with existing system. |  |  |  |
| All items | Requirements for Training ### Training courses shall be conducted on the equipment that will be delivered. All participants shall receive the documentation in native language in a book and electronic version. Training course should cover following functions: - Power user - Operator |  |  |  |
| All items | Warranty Warranty period 365 days from the issuance of PAC (Provisional Acceptance Certificate) in accordance with article 32 SC and GC of the Contract. Offer must include warranty service description including: - Service organisation contact data including name, postal address, telephone number, fax number and e-mail address; - Guaranteed that any requests for services will be attended to within 48 hours; - Guarantee that all items can be repaired or alternatively replaced within a maximum of 7 days; - Letter of confirmation that genuine spare parts and consumables will be available for a period of minimum 3 years from the date of final acceptance of the equipment. |  |  |  |
| All items | Commercial warranty 365 days from the issuance of FAC (Final Acceptance Certificate) in accordance with the conditions laid down in Article 32 of the General Conditions and Article 33 of the Special Conditions. Detailed description of the organisation of the proposed service and description of the Manufacturer’s commercial warranty shall be included in the offer. |  |  |  |

<Signature of authorised representative of the legal entity >

< Name and position of authorised representative of the legal entity >

< Date >